
DOUGLAS COUNTY


PRELIMINARY PLAT CHECKLIST

(DOUGLAS COUNTY CODE, APPENDIX B, ARTICLE V, SECTION 50)

Project Name:
_________________________________________________________________

Date Reviewed:
__________________________________

Reviewed By:
_________________________________________

Engineering Department

Please address all items marked with an "X" below, as well as any additional comments on the attached plan (s), prior to resubmittal.

______________________________________________________________________________

_____
The Preliminary Plat shall be clearly and legibly drawn at a standard engineering scale of not less than 1" = 100'.  Sheet size shall not exceed 36" x 48"; provided, however, that a scale of 1" = 200' may be used  in order to avoid sheets in excess of 36" x 48".  In no case may the sheet size be less than 8 1/2" x 11". 

_____
The Preliminary Plat shall be surveyed and drawn by a registered Professional Engineer, landscape Architect, or Land Surveyor, and the seal and signature of the person responsible for the plat shall be placed on the plat.

The Preliminary Plat shall contain the following:

_____
The proposed name of the subdivision, and the names of all proposed streets (letter designations may be used in place of street names on the Preliminary Plat).

_____
Name, street address, and telephone number of the owner of the property.

_____
Name, street address, and telephone number of the subdivider.

_____
Date of survey, north arrow, and graphic scale.

_____
Land lot, district, and section of Douglas County.

_____
Location sketch showing the subdivision in relation to the surrounding area with regard to well-known landmarks such as major roadways, railroads, streams, or others.  Scale should not be less than 1" = 2000'.

_____
Name of former subdivision if any or all of the property has been previously subdivided.

_____
Exact boundary lines of the tract indicated by a heavy line giving bearings and distances around the boundary.  The boundary lines shall include the entire tract, and all data required in this checklist shall apply to the entire property.

_____
Ground elevations on the tract based on field surveys or photogrammetric methods from aerial photography.  The datum for the topographic information shall be indicated on the plan; minimum  contour interval shall be 2'.  Note:  If the property is to be served by public sewer, or if the lot sizes are greater than one acre, the contour interval may be 5'.

_____
Natural features within the subdivision, including drainage channels, bodies of water, wetlands, and floodplains. 

_____
Right-of-way and pavement widths for existing and proposed streets.

_____
Location of existing bridges, culverts, and other features.

_____
Proposed layout of subdivision, including:

* Lot lines with dimensions;

* Sidewalks, where required;

* Lot numbers and block letters;

* Distance between cul-de-sacs;

* Proposed street names;  and

 

* Public-use easements.
_____
Stages or phases of development, if appropriate.

_____
Notation regarding sanitary sewer (public/septic).

_____
Location and size of existing and proposed sanitary sewer lines and water lines, and proposed location of fire hydrants.

_____
Minimum Lot Size – 35 % slope/100-year floodplain/other unsuitable land

_____
Other

              _______________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

​​​​​​​​​​​​​​​​​​​​​______________________________________________________________________________
